gedit barrel_shifter_isolated.v

`timescale 1ns/1ps

module barrel_shifter_isolated (

input  wire [7:0] data_in,

input  wire [2:0] shift,

output wire [7:0] data_out,

output wire [7:0] stage0_out,

output wire [7:0] stage1_out,

(* keep = "true", dont_touch = "true" *) output wire [7:0] shifted0_gated,

(* keep = "true", dont_touch = "true" *) output wire [7:0] shifted1_gated,

(* keep = "true", dont_touch = "true" *) output wire [7:0] shifted2_gated

);

wire [7:0] shifted0_raw, shifted1_raw, shifted2_raw;

// ---- Stage 0 (x1) ----

assign shifted0_raw = {data_in[6:0], 1'b0};

assign shifted0_gated = {8{shift[0]}} & shifted0_raw;

assign stage0_out     = shift[0] ? shifted0_gated : data_in;

// ---- Stage 1 (x2) ----

assign shifted1_raw = {stage0_out[5:0], 2'b00};

assign shifted1_gated = {8{shift[1]}} & shifted1_raw;

assign stage1_out     = shift[1] ? shifted1_gated : stage0_out;

// ---- Stage 2 (x4) ----

assign shifted2_raw = {stage1_out[3:0], 4'b0000};

assign shifted2_gated = {8{shift[2]}} & shifted2_raw;

assign data_out       = shift[2] ? shifted2_gated : stage1_out;

endmodule