gedit tb_barrel_shifter.v

`timescale 1ns/1ps

module tb_barrel_shifter;

reg  [7:0] data_in;

reg  [2:0] shift;

// ---- functional equivalence check ----

wire [7:0] conv_out, conv_s0, conv_s1;

barrel_shifter_conv dut_conv (

.data_in(data_in), .shift(shift),

.data_out(conv_out), .stage0_out(conv_s0), .stage1_out(conv_s1)

);

wire [7:0] iso_out, iso_s0, iso_s1, g0, g1, g2;

barrel_shifter_isolated dut_iso (

.data_in(data_in), .shift(shift),

.data_out(iso_out), .stage0_out(iso_s0), .stage1_out(iso_s1),

.shifted0_gated(g0), .shifted1_gated(g1), .shifted2_gated(g2)

);

// ---- un-gated reference versions of the same 3 nodes ----

wire [7:0] raw0 = {data_in[6:0], 1'b0};

wire [7:0] raw1 = {iso_s0[5:0], 2'b00};

wire [7:0] raw2 = {iso_s1[3:0], 4'b0000};

function integer pop;

input [7:0] v;

integer i;

begin

pop = 0;

for (i = 0; i < 8; i = i + 1) pop = pop + v[i];

end

endfunction

integer raw_t, gated_t, errors, s, rep;

reg [7:0] p_raw0, p_raw1, p_raw2, p_g0, p_g1, p_g2;

initial begin

$dumpfile("waveform.vcd");

$dumpvars(0, tb_barrel_shifter);

raw_t = 0; gated_t = 0; errors = 0;

data_in = 0; shift = 0;

p_raw0=0; p_raw1=0; p_raw2=0; p_g0=0; p_g1=0; p_g2=0;

#1;

for (s = 0; s < 8; s = s + 1) begin

shift = s[2:0];

for (rep = 0; rep < 256; rep = rep + 1) begin

data_in = rep[7:0];

#5;

if (conv_out !== iso_out) begin

errors = errors + 1;

$display("MISMATCH shift=%0d data_in=%b conv=%b iso=%b",

shift, data_in, conv_out, iso_out);

end

raw_t = raw_t + pop(raw0 ^ p_raw0) + pop(raw1 ^ p_raw1) + pop(raw2 ^ p_raw2);

gated_t = gated_t + pop(g0 ^ p_g0) + pop(g1 ^ p_g1) + pop(g2 ^ p_g2);

p_raw0 = raw0; p_raw1 = raw1; p_raw2 = raw2;

p_g0 = g0; p_g1 = g1; p_g2 = g2;

end

end

$display("--------------------------------------------------");

$display("Functional check: %0d mismatches (must be 0)", errors);

$display("Un-gated shift-branch toggles: %0d", raw_t);

$display("Gated (isolated) shift-branch toggles: %0d", gated_t);

$display("Switching-activity reduction on isolated nodes: %0.1f %%",

100.0 * (raw_t - gated_t) / raw_t);

$display("--------------------------------------------------");

if (errors != 0) begin

$display("FAIL: functional mismatches detected");

$fatal(1);

end else begin

$display("PASS: designs are functionally equivalent");

end

$finish;

end

endmodule